Code: A-28           Subject: COMPUTER NETWORKS
Time: 3 Hours                                                     June 2006                                                 Max. Marks: 100 
 
NOTE: There are 9 Questions in all.
      Question 1 is compulsory and carries 20 marks. Answer to Q. 1. must be written in the space provided for it in the answer book supplied and nowhere else.
      Out of the remaining EIGHT Questions answer any FIVE Questions. Each question carries 16 marks.
      Any required data not explicitly given, may be suitably assumed and stated.
 
 
Q.1       Choose the correct or best alternative in the following:                                         (2x10)
        
a.       The _________ layer changes bits into electromagnetic signals.
 
                   (A)  physical                                        (B)  data link
(C)    transport                                      (D)  none of (A), (B) and (C)
        
b.      ________switching uses the entire capacity of a dedicated link  
 
(A)    circuit                                           (B) message
(C) virtual circuit                                  (D) datagram  
                                                                                                                                             
             c.   In an asynchronous TDM, for N signal sources, each frame contains M slots, where M is usually___________N.
                   
(A)    equal to                                        (B)  greater than
(C)  less than                                       (D)  one less than
 
             d.   HDLC________field defines the beginning and end of a frame.
 
(A)    flag                                             (B)  address
(C)  control                                         (D)  FCS   
 
             e.   ________is the access protocol used by the conventional Ethernet.
                   
(A)     CSMA                                        (B)  CSMA/CD
(C)  CSMA/CA                                  (D)  Token ring
 
             f.    Stations do not sense the medium during_____time.
 
(A)     RTS                                             (B)  CTS
(C)  SIFS                                            (D)  NAV
 
             g.   Which OSI layer corresponds to the IP layer?
 
(A)     physical                                        (B)  data link
(C)  transport                                      (D)  network
 
 
 
             h.   In______ARQ, if a NAK is received, only the specific damaged or lost frame is retransmitted.
 
(A)    Stop and wait                               (B)  selective repeat
(C)  go-back-N                                   (D)  (A) & (B)
 
             i.    The VPI of an NNI is________bits in length.
 
(A)  8                                                  (B)  12
(C)  16                                                (D)  24
 
             j.    Digital signature can provide _____for a network. 
 
(A)   Authentication                               
(B)   Integrity
(C) Non-repudiation
(D) All of the (A), (B) and (C)
 
 
Answer any FIVE Questions out of EIGHT Questions. 
Each question carries 16 marks.
 
  Q.2     a.   Make a comparison of OSI reference model with TCP/IP reference model.       (8)
        
             b.   Suppose two new devices are added to an existing 6 devices network. If  the network has a mesh topology, how many new cable links are needed? How many new cable links will be needed if the network has a ring topology?                                                                                                          (4)
             
             c.   With suitable diagrams and data organized in time slots, explain how synchronous TDM is different from asynchronous TDM.                           
